# DURA BIKE LOCKER 3-PART SPECIFICATION

# PART 1 – GENERAL

  - SUMMARY

- This section includes the following:

- HD High Security Outdoor Bicycle Pump

  - QUALITY ASSURANCE

- Installer Qualifications: An experienced installer who has completed installation of similar bicycle parking racks.

- Manufacturer Qualifications: Graber Manufacturing, Inc. is a skilled manufacturer of bicycle parking racks since 1989.

- Source Limitations: For consistent quality in appearance and physical properties obtain each product from Graber Manufacturing, Inc.

- Product Options: Drawings indicate size, shape and dimensional requirements of furnishings for the purpose of customer sourcing.

  - SUBMITTALS

- Product Data: Include physical characteristics such as materials, specifications and finish.

- Drawings: Show details including dimensions, materials, and options for each product.

- Samples for Verification: Finish samples of the color and type of powder coat will be provided upon request.

- Maintenance Data: For each product include the recommended methods for repairing damage to the powder coat and materials finish will be provided.

  - DELIVERY, STORAGE AND HANDLING

- Store product(s) in original undamaged packaging and containers until ready for installation.

- Handle powder coated product(s) with sufficient care to prevent any scratches or damage to the finish.

  - WARRANTY

- Bicycle lockers carry a one year manufacturer’s limited warranty against defects in materials and workmanship. The warranty period begins the date of invoice.

# PART 2 - PRODUCTS

  - ACCEPTABLE MANUFACTURERS

- Dura Bike Locker, a division of Hannan Specialties, Inc. 3790 Bradview Dr., Sacramento, CA 95827Toll free phone: 800-722-2453

  - MATERIALS

- High Security Outdoor Public Bike Pump

- Main Body .25” and .125” 304 stainless steel

- Pump shaft:  ¾” 304 stainless steel

- Pressure gauge cover: ¼” polycarbonate.

- Integrated air hose: High pressure rubber exterior hose with triple braided steel wire core

- Optional Tools  tethered by 3/16” stainless steel cable

  - Steel core tire lever (qty 2.)

  - 15mm box wrench

- High Security Indoor Public Bike Pump

- Main Body .25” and 7 gauge steel

- Pump shaft:  ¾” 304 stainless steel

- Pressure gauge cover: ¼” polycarbonate.

- Integrated air hose: High pressure rubber exterior hose with triple braided steel wire core

- Optional Tools  tethered by 3/16” stainless steel cable

  - Steel core tire lever (qty 2.)

  - 15mm box wrench

- Electric Bike Pump

- Main Body:  16 gauge steel

- Pressure gauge cover: ¼” polycarbonate.

- Integrated air hose: High pressure rubber exterior hose with triple braided steel wire core

- Air Control Panel

- Main Body:  16 gauge steel

- Pressure gauge cover: ¼” polycarbonate.

Integrated air hose: High pressure rubber exterior hose with triple braided steel wire core

# PART 3 - EXECUTION

3.1 EXAMINATION

Examine footprint to ensure conditions are adequate per architect specifications and plans. Do not proceed with installation if conditions are not adequate for install. Notify project architect if conditions are unsatisfactory.

3.2 PREPARATION

Make sure the location of the racks is flat, level, square, accurate for alignment, and correctly located for install of bicycle racks.

3.3 INSTALLATION

Surface Mount: Freestanding – concrete filled

Surface Rail Mount: An above ground rail that allows for the mounting of 2, 3 or 4 bikes and has the option to adjoin rails for additional parking.

3.4 QUOTING

- Product specification: Make sure the product item, finish options, fasteners, and expected lead time have been included in estimate.

- Pricing: Estimates are good for 45 days; contact a Dura Bike Locker representative for a current price list.

3.5 FREIGHT

- The (  ) can be shipped UPS and in some cases ship freight, dependent on quantity order. Call Dura Bike Locker at 1-800-722-2453 for a freight quotes.

3.6 PURCHASING

- Product specification: Make sure the product item, finish options, fasteners, and expected lead time have been included in estimate.

- Pricing: Estimates are good for 45 days; contact a Ground Control Systems representative for a current price list if estimate has expired.

- Placing an order: You can order via phone 1-800-722-7425; email – info@durabikelocker.com. Or, if submitting purchase orders, mail to: 3790 Bradview Dr., Sacramento, CA 95827